# 02 Carbon Accounting Annex (v2)

## Document Control

| Document Property | Details |
| --- | --- |
| Version | 2.0 (Audit-Ready) |
| Date | 2026-02-20 |
| Author | Ecological Nest Consulting Team (GAB Climate Smart Ltd) |
| Reviewer | Technical Lead / Forest Biometrician |
| Approval | [INSERT APPROVER STRUCTURE] |

# 1. Pool Justification and Emission Factors

In alignment with IPCC 2013 Wetlands Supplement and Verra VM0033, Above-Ground Biomass (AGB), Below-Ground Biomass (BGB), and Soil Organic Carbon (SOC) up to 100cm are included. Deadwood and litter are conservatively excluded.

# 2. Allometric Equations

Species-specific, regionally appropriate allometric equations are applied to minimize uncertainty. The primary species in the Volta Estuary and Greater Amanzule regions are Rhizophora mangle and Avicennia germinans.

## 2.1 Rhizophora mangle

AGB (kg) = 0.128 * (DBH^2.60)  [Reference: Komiyama et al., 2005 or equivalent validated equation]

## 2.2 Avicennia germinans

AGB (kg) = 0.140 * (DBH^2.40)  [Reference: Default robust mangrove equation]

# 3. Bulk Density Lab Protocol

Accurate determination of soil bulk density is critical for SOC calculations. The following strict protocol applies:

1. Extract core samples using standard cylinder dimensions to maintain fixed sample volumes.

2. Transfer samples to the laboratory maintaining cold chain-of-custody.

3. Dry the intact core samples in a forced-air oven at exactly 60°C for 72 hours until a constant weight is achieved.

4. Weigh immediately upon removal to prevent moisture re-absorption.